Monitoring ONEAXI – rev. 2

Contents

1	SITE CONFIG PRTG	1

1.1	General settings	1

1.2	Remote Site TO Internet DMM latency	1

1.3	Remote Site TO VPN DMM latency	2

1.4	Remote Site Bandwidth IN USE	3

2	PRTG config DMM	5

3	GRAFANA CONFIG	7

# SITE CONFIG PRTG

## General settings

Monitor type:

- ICMP Ping

  - Latency

- SNMPv2C

  - Bandwith in use

Monitoring interval: 30s

Test IP address in Lux: 82.143.66.193 (This is the Telenet router IP address in DMM)

## Remote Site TO Internet DMM latency

New sensor for ONEAXI (DEVICE = DMM INTERNET)

## Remote Site TO VPN DMM latency

New sensor for ONEAXI

## Remote Site Bandwidth IN USE

Existing sensor (new standard)

# PRTG config DMM

DMM-PRTG02 > Capellen > ONEAXI network monitoring

# GRAFANA CONFIG

Login: http://vm-sql-grafana:3000 (always use Chrome, credentials can be found in RDM)

Configuration – Data Sources – Add data source

Fill in name, type, URL (= https prtg url + ‘/api’), access mode = proxy

Unfortunately, due to a self-signed certificate on most PRTG nodes, we need to enable the “skip TLS verification” until further notice.

Username = prtgadmin (or another admin use)r

Passhash = passhash of the PRTG admin user (= NOT the password, see further)

Enable “timezone auto-adjust”

To find out the passhash of a user: login to prtg

Setup – system administration - user accounts

Select a user

“Show passhash”

While viewing the prtgadmin properties, scroll down and check the timezone settings of the user

Should be “UTC +1 Brussels”, else you will see gaps in the grafana graphs